Intelligent Speed Adaptation (ISA)

### Result of ISA from UK research and Swedish trial

1. Road safety

- UK research

| System type | Speed limit type | Best estimate of slight accidents reduction | Best estimate of fatal & serious accidents reduction |
| --- | --- | --- | --- |
| Mandatory | Fixed | 20% | 29% |
|  | Variable | 22% | 31% |
|  | Dynamic | 36% | 48% |

Table 1. Positive effect of road accidents

Advisory : display the speed limit and remind the driver of changes in the speed limit.

Voluntary (“Driver-Select”): allow the driver to enable and disable control by the vehicle of

maximum speed.

Mandatory: the vehicle is limited at all times

Fixed: the vehicle is informed of the posted speed limits

Variable: the vehicle is additionally informed of certain locations in the network where a lower

speed limit is implemented

Dynamic: additional low speed limits are implemented because of network or weather

conditions, to slow traffic in fog, on slippery roads, around major incidents, etc. With a

Dynamic system, speed limits are current in terms of time.

(2) Swedish trial

• Individual logging of vehicle speeds indicates an effect of 10-15% less injuries.

• Influence on non–ISA vehicles (average speed twice as affected as expected

from ISA vehicles alone) in Umeå indicates a greater effect.

• Improved interaction with vulnerable road users at intersections also indicates

a greater effect.

• Total effect in Umeå indicates 30–40% less injuries if all vehicles were

equipped with ISA. The first ISA vehicles had the greatest effect on non-ISA

vehicles.

2. Fuel consumption and traveling time

(1) UK research

| Road type |  | Fuel saving |
| --- | --- | --- |
| Motor way | All | 1% |
| Major roads | Non built up | 3% |
|  | Built up | 8% |
| Minor roads | Non built up | 3% |
|  | Built up | 8% |
| Overall |  | 5.5% |

Table 2.

Effect of fuel consumption and traveling time

(2) Swedish trial

Fuel consumption is believed to have dropped a little for informative, but only

marginally for warning and active gas. Travel times were unchanged (a

marginal improvement was even found) despite the lower maximum speed.

This is explained by fewer stops and braking situations with ISA. The drivers

were skilful in adapting to this new technology and contributed to this result.

3. Level of pollutants

- UK research

Level of pollutants did show great change. However, it was not show negative effect.

- Swedish trial

Carbon monoxide emissions decreased by 11%

Nitric oxide emissions decreased by 7%

Hydrocarbon emissions decreased by 8%

Carbon dioxide emissions decreased by 1%