Data for Thermal Solar Energy System

Address:		Mabie Farm Park		Contact: Alan Shuttleworth

Mabie,				Tel: 01387259666

Dumfries

DG28EZ

Building types:

- Visitor Centre/Tearoom and Outdoor Toilet

- Single Storey

- Modern builds (post 2000)

- Well insulated: - 0.36m thick walls

- rock wool insulation

- double glazed windows and doors

Estimated/measured hot water consumption (eg. litres per person/day):

- Farm Park is closed from November through to January

- Open weekends only February and March

- Open 10am -5pm 7 days a week April to October

Visitor numbers vary and are dependent on a number of factors such as

time of year, weather, school holidays, school trips etc. Peak visitor

numbers during summer can be up to 1000 people per day at weekends.

An average of 3 litres per person per day is assumed for hand

washing, cleaning etc.

Other hot water requirements (eg. washing machine, dishwasher or agricultural use):

- One dishwasher (using 20 litres per wash)

- No agricultural use of hot water

- No washing machine on business premises

Type of installation (eg. in roof, on roof, free standing, flat roof, façade/wall/balustrade):

- On roof

Roof covering (eg. tiles, stone tiles, slates, other?):

- Slates (stable, modern build)

Angle, alignment (South facing), height from ground:

- Roof angle is 30o

- South facing roof on Visitor Centre

- Roof is approximately 2.5 metres from ground level

Usable installation area (m2):

- Usable installation area on south facing roof is 25 m2

Shade:

- No obvious sources of shade for south facing roof

location on visitor centre

- Climbing plant which can be cut back

- Hillside to south and west which would slightly decrease

Daylight hours

Location/space (m) for solar store installation:

- Utility area directly below proposed solar collector site

So adequate space for solar store

Work/distance required for connection pipe from store to collector:

- Minor work

- Between 2 to 3 metres

Existing hot water stores/boilers (type, make, volume and power):

- Pre-existing 60 litres hot water storage

- Modern good efficiency combi boiler

Solar Data (Direct/Diffuse Radiation):

- Solar data available from,

http://re.jrc.ec.europa.eu/pvgis/apps/radmonth.php?en=&europe=

Any other useful information: