Data for Thermal Solar Energy System

Address:		Glenquicken House		Contact: Miles Wenner

Creetown,

Dumfries and Galloway

Email:	mileswenner@developingforestry.freeserve.co.uk

Building types:

- Traditional Swedish Farm House

- Two stories

- Well insulated:  - rock wool insulation

- double glazed windows and doors

Estimated/measured hot water consumption (eg. litres per person/day):

- Average hot water consumption of 50 litres per person/day

- Average occupancy is 2 people

Other hot water requirements (eg. washing machine, dishwasher or agricultural use):

- One dishwasher (using 20 litres per wash)

- No agricultural use of hot water

- Washing Machine (using 30 litres per wash)

Type of installation (eg. in roof, on roof, free standing, flat roof, façade/wall/balustrade):

- On roof

Roof covering (eg. tiles, stone tiles, slates, other?):

- Green roof (sedum moss)

Angle, alignment (South facing), height from ground:

- Roof angle is 40o

- South facing roof

- Roof is approximately 3-5 metres from ground level

Usable installation area (m2):

- Usable installation area on south facing roof is 20-30 m2

Shade:

- No obvious sources of shade for south facing roof

location

Location/space (m) for solar store installation:

- Utility area within building provides adequate space for

500 litres hot water storage

Work/distance required for connection pipe from store to collector:

- Minor work

- Possible complications with green roof

- Between 4 to 6 metres

Existing hot water stores/boilers (type, make, volume and power):

- Pre-existing 500 litres hot water storage

- Large wood burning furnace for space heating

Solar Data (Direct/Diffuse Radiation):

- Solar data available from,

http://re.jrc.ec.europa.eu/pvgis/apps/radmonth.php?en=&europe=

Any other useful information:

- The house is still being built. Was disassembled in Sweden and was 		   transported to the UK.

- Green roof may require additional work for solar collector 			   installation.