BASESIMP models the heat loss to the ground through either the basement or the slab. This document will identify the three modifications required to the standard ESP-r/HOT3000 input files for BASESIMP simulation.

The required inputs are defined in BASESIMP_Input_File_Format.pdf (available on the HOT3000 Documentation FTP site).

# Format of the BASESIMP (*.bsm) Input File

BsHEIG	# Foundation height

BsDEPT	# Foundation depth

bsLENG 	# Foundation length

bsWIDT	# Foundation width

bsOVER 	# Insulation overlap for "combination" configurations

bsRSI 		# Insulation resistance in RSI

soilK		# Soil conductivity

bsWTD 	# Water-table depth

bsTGavg 	# Annually-averaged soil temperature

bsTGamp 	# Amplitude of ground-temperature's annual sine wave

bsTGps 	# Phase lag of ground-temperature's annual sine wave

Notes:

- For the case of the BASESIMP simulation only, bsLENG ≥ bsWIDT.

- The maximum values of bsLENG and bsWIDT are 12m.

  - Basement Geometry

The height, width, length, and depth of the basement or slab required for the BASESIMP model will be taken directly from the HOT2000 interface.

  - Wall Insulation

The RSI of the basement wall (bsRSI) is defined as the additional insulation added to the foundation framing, if applicable.

Section 2 describes the definition of the foundation multi-layer construction.

  - Soil Properties

The following soil properties are required for the BASESIMP model: soil conductivity (bsSOIL), water-table depth (bsWTD), annually-averaged soil temperature (bsTGavg), amplitude of ground-temperature's annual sine wave (bsTGamp), and phase lag of ground-temperature's annual sine wave (bsTGps).

For the H3K_Web project, these values were calculated using the Moore Method, based on the cities average monthly outdoor dry bulb temperature and degree-days. These values are defined in the database, based on the user-input house location (city).

For HOT3000, these values can be calculated.

# The Construction File Format (*.con)

This section will describe the mapping of the foundation construction to both the .con file and the .bsm file.

Take for example the following BASESIMP configurations:

BCIN_1

- interior surface of wall insulated over full-height

- any first-floor construction type

- 200mm concrete walls and 100mm concrete floor

- Use column 3 of Table 1 for the corner-correction method (Beausoleil-Morrison et al 1995b)

BCNN_2

- no insulation

- first-storey is non-brick veneer or bricks thermally broken from basement’s concrete walls

- 200mm concrete walls and 100mm concrete floor

- Use column 1 of Table 1 for the corner-correction method (Beausoleil-Morrison et al 1995b)

In the construction input file (.con), these two configurations will be modelled identically, it is only in the BASESIMP input file that the differences in construction will be defined. For BCIN_1, the value of bsRSI will reflect the additional insulation added to the interior of the concrete wall, whereas for BCNN_2, bsRSI = 0.0.

In the HOT2000 interface, there is a input field for the added insulation (interior or exterior) for walls and slab. It is this RSI value that will be mapped to bsRSI.

In the event that RSI values for the wall and slab constructions are not the same, an effective RSI value should be used.

# The Connection File Format (*.cnn)

The mapping of the connections file is currently as was specified for the H3K_web project. The only modification needed is to map the BASESIMP configuration specified in the interface to the value in the 4th column of the .cnn file.

The following table maps the user-defined BASESIMP configuration to the value required in the .cnn file.

| 1 | BCIN_1 | 26 | BCEB_2 | 51 | SCB_24 | 76 | BCEB_8 | 101 | BCEB_6 | 126 | BCEB_10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | BCIN_2 | 27 | BCEB_3 | 52 | SCB_25 | 77 | BCEB_9 | 102 | BWIA_1 | 127 | BCEB_11 |
| 3 | BCIN_3 | 28 | SCN_1 | 53 | SCB_26 | 78 | BWEN_3 | 103 | BWIA_2 | 128 | BCEA_3 |
| 4 | BCIN_4 | 29 | SCN_2 | 54 | SCB_29 | 79 | BBIB_3 | 104 | SCA_1 | 129 | BCEA_9 |
| 5 | BCEN_1 | 30 | SCN_3 | 55 | SCB_30 | 80 | BBIB_4 | 105 | SCA_2 | 130 | BCEA_10 |
| 6 | BCEN_2 | 31 | SCN_4 | 56 | SCB_33 | 81 | SCB_31 | 106 | SCA_9 | 131 | BCEA_11 |
| 7 | BCEN_3 | 32 | SCN_7 | 57 | SCB_34 | 82 | SCB_32 | 107 | SCA_10 | 132 | BWIB_1 |
| 8 | BCEN_4 | 33 | SCN_8 | 58 | SCB_35 | 83 | SCB_37 | 108 | BBIN_1 | 133 | BWIB_2 |
| 9 | BCNN_1 | 34 | SCB_1 | 59 | SCB_36 | 84 | SCB_38 | 109 | BCEN_5 | 134 | BWIB_4 |
| 10 | BCNN_2 | 35 | SCB_2 | 60 | SCA_17 | 85 | SCB_39 | 110 | BCEN_6 | 135 | BWIA_3 |
| 11 | BCCN_1 | 36 | SCB_3 | 61 | SCA_18 | 86 | SCB_40 | 111 | BBIA_1 | 136 | BWEB_1 |
| 12 | BCCN_2 | 37 | SCB_4 | 62 | SCA_19 | 87 | BBEB_1 | 112 | BBIB_1 | 137 | BWEB_2 |
| 13 | BWNN_1 | 38 | SCB_5 | 63 | SCA_20 | 88 | BBEN_1 | 113 | BBIB_2 | 138 | BWEB_3 |
| 14 | BWIN_1 | 39 | SCB_6 | 64 | SCA_21 | 89 | BBEN_2 | 114 | BCCB_9 | 139 | BWEB_4 |
| 15 | BWIN_2 | 40 | SCB_9 | 65 | SCA_22 | 90 | BBIA_2 | 115 | BCCB_10 | 140 | BBIB_5 |
| 16 | BWIN_3 | 41 | SCB_10 | 66 | SCA_23 | 91 | BBIN_2 | 116 | BCCA_1 | 141 | BBIB_6 |
| 17 | BWEN_1 | 42 | SCB_11 | 67 | SCA_24 | 92 | BCCB_8 | 117 | BCCA_4 | 142 | BBEB_3 |
| 18 | BWEN_2 | 43 | SCB_12 | 68 | BCCN_3 | 93 | BCCA_7 | 118 | BCIB_7 | 143 | BBEB_4 |
| 19 | BCIB_1 | 44 | SCB_13 | 69 | BCCB_4 | 94 | BCCA_8 | 119 | BCIB_8 | 144 | BBEB_5 |
| 20 | BCIB_2 | 45 | SCB_14 | 70 | BCEA_1 | 95 | BCCN_4 | 120 | BBEB_2 | 145 | BBEA_2 |
| 21 | BCIB_3 | 46 | SCB_17 | 71 | BCEA_4 | 96 | BCCN_5 | 121 | BCIA_3 |  |  |
| 22 | BCIB_4 | 47 | SCB_18 | 72 | BCIA_1 | 97 | BCEA_5 | 122 | BCIA_5 |  |  |
| 23 | BCIB_5 | 48 | SCB_21 | 73 | BCIA_4 | 98 | BCEA_6 | 123 | BCIA_6 |  |  |
| 24 | BCIB_6 | 49 | SCB_22 | 74 | BCEA_7 | 99 | BCEB_4 | 124 | BCIB_9 |  |  |
| 25 | BCEB_1 | 50 | SCB_23 | 75 | BCEA_8 | 100 | BCEB_5 | 125 | BCIB_10 |  |  |